Numerical modelling of vegetation and morphology evolution in New Zealand braided rivers.

Guglielmo Stecca1

1National Institute of Water and Atmospheric Research (NIWA), Christchurch, New Zealand

- This abstract is for an Oral

- Session: Numerical Models of Morphodynamics

# ABSTRACT

River planform results from the complex interaction between flow, sediment transport and vegetation, and can evolve following a change in these controls. Disentangling this complex causation path is not straightforward (Stecca et al., 2019). We propose a modelling approach (Stecca et al., 2022) that can be used as tool for analysis of observed trajectories and to forecast future behaviours in braided rivers impacted by exotic vegetation under different hydrological regimes. Predicting these changes is relevant both to assessing the environmental quality of the riverine physical environment, and for multi-hazard assessment, as the river’s morphological evolution controlled by vegetation presence and alterations of the flow regime can impact asset and land with increased risk of flooding and lateral erosion.

We focus two iconic braided river cases in New Zealand’s South Island: the Lower Waitaki River and the Waimakariri River. The Waitaki is impacted by the combined effects of exotic vegetation and a hydropower scheme that has altered the flow regime. As the Waitaki River is unable to clear vegetation efficiently, vegetation encroachment has promoted a shift towards a single-thread morphology. In contrast, the more active Waimakariri River, despite having been subjected to similar vegetation, retains a largely unvegetated channel due to its ability to naturally clear vegetation.

Calibration and validation of the model’s vegetation parameter settings, which is critical to obtaining realistic planform styles, is carried out by selecting the parameter values that allow the model to predict vegetation encroachment in the Waitaki and efficient vegetation clearing in the Waimakariri. The model responds sensibly to changes in parameters, showing that more aggressive vegetation types cause a sharper reduction of braiding (Stecca et al, in press).

Finally, we study summary metrics of vegetation presence from simulations as a function of a synthetic parameter indexing the relative influence of hydrological and vegetation controls. We find that vegetation presence across rivers and flow regimes can be explained by the the duration of periods of vegetation growth, compared with the characteristic timescale of such vegetation growth.

# REFERENCES

Stecca, G., Zolezzi, G., Hicks, D. M., & Surian, N. (2019). Reduced braiding of rivers in human-modified landscapes: Converging trajectories and diversity of causes. Earth-science reviews, 188, 291-311.

Stecca, G., Fedrizzi, D., Measures, R. Hicks, D. M., Hoyle, J. & Zolezzi, G. (2022). Development of a numerical model for braided river morphology and vegetation evolution with application to the Lower Waitaki River (Aotearoa-New Zealand). Advances in Water Resources, 166, 104236.

Stecca, G., Hicks, D. M., Measures, R., Henderson, R. (in press). Numerical modeling prediction of vegetation trajectories under different flow regimes in New Zealand braided rivers. Journal of Geophysical Research: Earth Surface, 128, e2023JF007397.